To connect your Yealink device with ADB, you will need files:

- ADBkey_1.0.0.1

- Adb_tools

- Encrypted mac.cfg file from Yealink (please send MAC address list to Yealink to obtain one)

- .andriod .zap if you cannot find it under user file.

- Install ADBkey_1.0.0.1 on your PC. When installing, a .andriod file will be created under user file of your PC. Manually creation will not work.  If you cannot find it, unzip the .andriod file in the package to your user profile.

- Report your MAC address to Yealink to get an encrypted mac.cfg

- Upgrade firmware to 58.80.0.201 which has ADB feature enabled by default.

- Auto provisioning the encrypted mac.cfg to your device. Note that auto provision need to be done every time you reboot the phone.

Auto provision parameter: adb_permission.url=http://10.91.80.53:8080/T58V.cfg   ##the URL is point to the secured mac.cfg which you get from Yealink.

- Unzip and install attached adb_tools to your PC (I have it stored under user profile)

- Use DOS windows to run the adb_tools and connect to your Yealink device: